This is for a 16 ft Runabout with 3-4 Stringers of approx 6-8” depth, 1 ½” transom thickness and a ¾” thick Deck.   It also assumes using ¾” plywood for all components i.e. Deck, Stringers and Transom.

These are Guess-Ti-Mates but should get you close.

PPE (Personal Protection Equipment)

Respirator (Full Face Preferred $100) if not then double filtered 1/2 face and Goggles with vents taped over.
Latex Gloves (1 Box)
Leather Gloves 1 Pair
Full cover Hooded Tyvek suit.
Talcum Powder (coat yourself TOTALLY with talcum powder prior to every grinding Session.  USE A LOT) It seals the pores of your skin and REALLY helps keep off the ITCH!!!!

Demolition Equipment

Prybars, Hammers, chisels, Putty Knives
Flat tipped shovel to remove foam (if you have any)
Circular Saw
Sawzall
Dremel Tool
Router
Drill
Random Orbit Sander
Rasps, Files
4 1/2" Grinder with Backup Pad and 24 grit resin coated sanding discs
Masking Tape

Restoration Products

3-4 sheets 3/4" Arauco Ext. Grade Plywood
15 Gallons Polyester Laminating Resin
4 Gallons Cabosil/Aerosil
1 lbs 1/4" chopped strand fibers
20 yds 1.5 oz CSM
15 yds 1708 Biaxial Cloth
dozen 1 qt plastic mixing buckets
1/2" x 5" Fiberglass ”Bubble Buster” roller
Short nap 6" nylon roller covers
2 dozen 3" Chip Brushes